AUSHADHI VAULT

﻿
﻿
Project Title: AI-Powered Smart Medicine Dispensing System﻿


1. Project Overview﻿

Aushadhi Vault is a proposed AI-powered smart medicine dispensing system designed to improve access to prescribed medicines in rural and semi-urban communities. In many areas, people may face difficulties accessing pharmacies, qualified healthcare professionals, and reliable medicine services. Aushadhi Vault aims to address this last-mile healthcare access challenge by combining technology, prescription processing, professional verification, and automated dispensing in a single system.﻿

The proposed system consists of a physical kiosk equipped with a touchscreen, camera, medicine storage compartments, an automated dispensing mechanism, and an integrated software platform. It is designed to guide users through prescription submission, AI-assisted prescription reading, safety checks, remote consultation when needed, pharmacist verification, and controlled medicine dispensing.﻿

The system uses a Raspberry Pi 5 with 8 GB RAM as the main computing unit. An ESP32-S3 microcontroller can manage communication with sensors and dispensing actuators. A camera or scanner captures prescription images, while Optical Character Recognition (OCR) converts the images into machine-readable text. AI and Natural Language Processing (NLP) techniques can help organize extracted information and identify missing or inconsistent details.﻿

The proposed system may also include drug-interaction alerts using a reliable drug-information source, teleconsultation facilities, inventory monitoring, RFID-based staff access, and dispensing confirmation using sensors and a load cell.﻿

A crucial feature of Aushadhi Vault is its professional verification process. AI assists with reading prescriptions and flagging possible issues, but it does not independently prescribe medicines or authorize dispensing. An authorized pharmacist must review and approve the prescription wherever required before the system releases the medicine.﻿

The long-term objective is to develop a reliable, accessible, and safe technology platform that supports medicine availability and improves the user experience in underserved communities.﻿

2. Problem Statement﻿

Access to medicines is an important part of healthcare, but obtaining the right medicine can be difficult when a pharmacy or qualified healthcare professional is far away.﻿

Some of the challenges the project seeks to address include:﻿

* Limited convenient access to pharmacies in certain rural and semi-urban locations.﻿

* Delays in obtaining prescribed medicines.﻿

* Manual prescription-reading difficulties, particularly with unclear handwriting.﻿

* Limited access to remote medical consultation.﻿

* Medicine inventory and availability challenges.﻿

* The need for verification before medicine dispensing.﻿

* Limited visibility into dispensing records and stock levels.﻿

Aushadhi Vault proposes an integrated technology-based approach to these challenges. Its purpose is to support access and improve the dispensing workflow, not to replace doctors, pharmacists, or emergency medical services.﻿

 3. Objectives of Aushadhi Vault﻿

* To study and design a smart medicine dispensing kiosk.﻿

* To explore OCR and AI-assisted prescription processing.﻿

* To establish a verification workflow involving qualified healthcare professionals.﻿

* To investigate teleconsultation integration for remote medical guidance.﻿

* To design an automated dispensing mechanism using electronic components.﻿

* To monitor medicine inventory and dispensing events.﻿

* To incorporate sensors for detecting dispensing errors and hardware failures.﻿

* To improve accessibility through a simple touchscreen interface.﻿

* To investigate privacy, security, and applicable healthcare regulations.﻿

* To develop a structured prototype and testing plan.﻿

4. How the Complete System Works﻿

The following is the proposed end-to-end workflow of Aushadhi Vault.﻿

1. User starts﻿

The user interacts with the kiosk touchscreen.﻿

2. Prescription upload﻿

A camera or scanner captures the prescription.﻿

3. OCR and AI processing﻿

Prescription text is extracted and organized.﻿

4. Validation and safety checks﻿

Missing, unclear, or potentially risky details are flagged.﻿

5. Teleconsultation, if required﻿

The user can be connected to a qualified healthcare professional.﻿

6. Pharmacist verification﻿

An authorized pharmacist reviews and approves, rejects, or requests clarification.﻿

7. Medicine availability check﻿

The system checks the relevant inventory record.﻿

8. Automated dispensing﻿

The controller operates the dispensing mechanism after authorization.﻿

9. Dispensing confirmation﻿

Sensors may detect movement, stock changes, or mechanism errors.﻿

10. Record and session closure﻿

The system updates inventory and records the transaction.﻿

Step 1: User interaction﻿

The user begins by interacting with the touchscreen. The interface provides instructions for submitting a prescription and explains how the information will be processed.﻿

Step 2: Prescription capture﻿

The user scans or uploads a prescription through the camera or scanner. The system checks image quality and requests another image if the prescription is blurred, incomplete, or difficult to read.﻿

 Step 3: OCR and AI-assisted processing﻿

OCR converts the prescription image into text. The software attempts to identify information such as:﻿

* Medicine name.﻿

* Medicine strength.﻿

* Dosage and frequency.﻿

* Duration of use.﻿

* Prescription date and prescriber information, where available.﻿

The system must flag uncertain results instead of guessing the intended medicine or dosage.﻿

 Step 4: Validation and safety checks﻿

The software checks whether essential information is missing, whether medicine names are ambiguous, and whether configured safety rules identify potential concerns. Drug-interaction alerts would require integration with an appropriate, reliable reference source.﻿

Any uncertainty or high-risk alert must be sent for professional review.﻿

Step 5: Teleconsultation﻿

If clarification or medical advice is needed, the system can facilitate a remote consultation with a qualified healthcare professional. The workflow must account for internet failures and unavailable clinicians.﻿

 Step 6: Pharmacist verification﻿

An authorized pharmacist reviews the prescription, extracted information, and relevant alerts. The pharmacist can approve, reject, or request clarification. The system records the decision and the authorized person's identity.﻿

Step 7: Medicine availability﻿

After the required approval, the system checks whether the medicine is available in the mapped storage compartment. If it is unavailable or stock information is inconsistent, dispensing is blocked.﻿

 Step 8: Automated dispensing﻿

The Raspberry Pi sends an authorized dispensing command to the ESP32-S3. The microcontroller operates the appropriate servo or actuator to release the selected medicine according to the designed mechanism.﻿

 Step 9: Dispensing confirmation﻿

Sensors and a load cell may help detect movement, weight changes, or dispensing failures. If the mechanism jams or sensor readings are inconsistent, the system stops and requests staff intervention.﻿

Step 10: Record keeping﻿

The system records the dispensing decision, medicine item, quantity, timestamp, inventory change, and any error that occurred. The user receives collection instructions, and the session closes securely.﻿

Safety principle: AI-assisted processing must never independently authorize medicine dispensing. Professional verification, access controls, and hardware safety checks are essential.﻿

 5. Components and Their Uses in Aushadhi Vault﻿

5.1 Raspberry Pi 5 (8 GB RAM)﻿

Purpose: Main computing unit.﻿

The Raspberry Pi 5 acts as the central computer of the kiosk. It coordinates the software workflow and connects the user interface with the prescription-processing and hardware-control systems.﻿

Uses:﻿

* Runs the main application and user interface.﻿

* Coordinates prescription upload and OCR processing.﻿

* Manages prescription status and pharmacist-review workflows.﻿

* Communicates with the ESP32-S3.﻿

* Connects software services and inventory records.﻿

* Records system events and error messages.﻿

5.2 ESP32-S3 Microcontroller﻿

Purpose: Electronic control and hardware communication.﻿

The ESP32-S3 is intended to manage low-level hardware functions independently of the main computer.﻿

Uses:﻿

* Reads connected sensors.﻿

* Controls servo motors or other compatible actuators.﻿

* Reports hardware status to the Raspberry Pi.﻿

* Handles defined dispensing commands.﻿

* Helps detect faults in the physical dispensing mechanism.﻿

5.3 Touchscreen Display﻿

Purpose: User interaction.﻿

The touchscreen allows users to operate the kiosk without requiring a separate computer.﻿

Uses:﻿

* Displays instructions.﻿

* Provides prescription upload controls.﻿

* Shows processing and verification status.﻿

* Displays medicine availability.﻿

* Guides users through collection.﻿

* Provides error messages and assistance instructions.﻿

5.4 Camera or Prescription Scanner﻿

Purpose: Prescription image capture.﻿

The camera or scanner captures the prescription for digital processing.﻿

Uses:﻿

* Captures prescription images.﻿

* Provides image input for OCR.﻿

* Allows users to rescan unclear documents.﻿

* Supports human review of the original prescription.﻿

 5.5 OCR Software﻿

Purpose: Convert prescription images into text.﻿

OCR stands for Optical Character Recognition. It identifies printed or handwritten text in an image, although handwritten prescription recognition can be unreliable.﻿

Uses:﻿

* Extracts medicine names and prescription details.﻿

* Reduces manual data entry.﻿

* Provides structured information for further review.﻿

* Flags low-confidence or unreadable fields.﻿

5.6 AI and NLP Processing﻿

Purpose: Organize and assess extracted information.﻿

AI and NLP can help interpret OCR output and identify missing or inconsistent fields.﻿

Uses:﻿

* Organizes medicine and dosage information.﻿

* Identifies incomplete prescription records.﻿

* Flags ambiguous medicine names.﻿

* Supports configured drug-interaction alerts.﻿

* Sends uncertain cases for human review.﻿

These features require validation against reliable data. AI must not independently prescribe or authorize medicine.﻿

5.7 Teleconsultation Module﻿

Purpose: Remote healthcare consultation.﻿

Uses:﻿

* Helps connect users with qualified healthcare professionals.﻿

* Allows relevant information to be shared with consent.﻿

* Supports clarification when a prescription is unclear.﻿

* Provides a remote consultation pathway where local access is limited.﻿

5.8 Pharmacist Verification Interface﻿

Purpose: Professional review and authorization.﻿

Uses:﻿

* Displays the prescription and extracted information.﻿

* Shows relevant validation alerts.﻿

* Allows authorized pharmacists to approve, reject, or request clarification.﻿

* Records the decision and reviewer.﻿

* Prevents dispensing without the required approval.﻿

 5.9 RFID Module﻿

Purpose: Staff identification and access control.﻿

RFID means Radio-Frequency Identification.﻿

Uses:﻿

* Identifies an authorized staff card.﻿

* Supports restricted access to pharmacist or maintenance functions.﻿

* Can help distinguish routine user actions from staff-only actions.﻿

RFID alone should not be treated as complete security. Appropriate account authentication and permissions are also required.﻿

5.10 Servo Motors and Dispensing Mechanism﻿

Purpose: Controlled physical release of medicines.﻿

Uses:﻿

* Operates a dispensing gate, tray, or release mechanism.﻿

* Releases the approved item according to the configured design.﻿

* Works with the controller and sensor feedback.﻿

* Supports testing of automated dispensing.﻿

 5.11 Medicine Storage Compartments﻿

Purpose: Organized medicine storage.﻿

Uses:﻿

* Store medicines in mapped compartments.﻿

* Connect physical stock to inventory records.﻿

* Support restocking and stock reconciliation.﻿

* Help prevent selection of an incorrect compartment.﻿

Actual medicines require appropriate storage conditions and regulatory review.﻿

5.12 Sensors﻿

Purpose: Monitor physical operations.﻿

Depending on the selected sensor type, the system may use sensors to detect door position, tray movement, compartment state, or collection-slot activity.﻿

Uses:﻿

* Detect whether a mechanism has moved.﻿

* Identify potential jams or incomplete dispensing.﻿

* Provide feedback to the controller.﻿

* Help the software distinguish a successful command from a failed operation.﻿

 5.13 Load Cell﻿

Purpose: Measure weight changes.﻿

A load cell converts a mechanical load into an electrical signal that can be measured by the system.﻿

Uses:﻿

* Measures the weight of a compartment or tray.﻿

* Helps estimate stock changes.﻿

* Supports dispensing-confirmation checks.﻿

* Identifies unexpected weight differences.﻿

It requires calibration and should not be the sole proof that the correct medicine was dispensed.﻿

 5.14 Connectivity and Network﻿

Purpose: Communication with remote services.﻿

Uses:﻿

* Supports teleconsultation.﻿

* Connects to remote software services when required.﻿

* Enables authorized data synchronization.﻿

* Supports software maintenance and updates.﻿

The kiosk needs safe behavior when the internet is unavailable.﻿

 5.15 Database and Audit Logs﻿

Purpose: Record management.﻿

Uses:﻿

* Stores inventory records.﻿

* Tracks prescription workflow status.﻿

* Records pharmacist decisions.﻿

* Logs dispensing attempts and errors.﻿

* Supports authorized stock monitoring and auditing.﻿

Patient data must be protected through appropriate access controls, secure storage, and retention policies.﻿

5.16 Power Supply and Enclosure﻿

Purpose: Reliable and protected hardware operation.﻿

Uses:﻿

* Supplies stable power to the computer, controller, screen, and actuators.﻿

Achievement: BIRAC–EYUVA﻿

Conclusion:﻿

Aushadhi Vault proposes an integrated approach to improving medicine access through technology. By combining prescription capture, OCR, AI-assisted validation, teleconsultation, pharmacist verification, automated dispensing, and inventory monitoring, the system aims to make the medicine-access workflow more convenient and traceable.﻿
The 12-week plan progresses from understanding the problem and studying the hardware to designing the software workflow, dispensing mechanism, integration, and testing procedures.﻿
The most important requirement is safety. Unclear prescriptions, missing authorization, uncertain inventory, and hardware failures must prevent dispensing until the issue is resolved. AI should assist qualified professionals rather than replace their judgment.﻿
Before deployment, the team should validate the prototype with qualified healthcare and pharmacy professionals, test it under controlled conditions, protect patient information, and confirm applicable legal and regulatory requirements.﻿

Aushadhi Vault — Smart Medicine. Stronger Communities.